cpu name:

yo

language name:

gurt

created by:

brandon gaona & zidan kazi

i pledge my honor that i have abided by the stevens honor system

=====================================================================

syntax:

all gurt instructions are:

- completely lowercase

- separated by spaces

- intentionally minimal

- designed for the yo 8-bit instruction format

add

performs register addition

- computes rs + rt and stores the result in rd

- format: add rd rs rt

- opcode: 00

sub

performs register subtraction

- computes rs - rt and stores the result in rd

- format: sub rd rs rt

- opcode: 01

load

loads a value from memory into a register

- computes address = rb + offset imm and loads mem[address] into rd

- format: load rd rb imm

- opcode: 10

store

stores a register value into memory

- computes address = rb + imm and writes rs into mem[address]

- format: store rs rb imm

- opcode: 11

=====================================================================

registers

the yo cpu has four general-purpose registers

r0 r1 r2 r3

each register is encoded using 2 bits:

r0 = 00

r1 = 01

r2 = 10

r3 = 11

=====================================================================

running the assembler

to assemble a gurt program:

- create a .gurt file that contains a .text section for instructions and a .data section for initial memory values.

- all instructions must be lowercase and space-separated.

- make sure all gurt instructions follow the formats listed in this manual

- (add, sub, load, store) and that all registers are written as r0–r3

- place your .gurt file in the same directory as assembler.py

- open a terminal in that directory and run:

- python assembler.py filename.gurt

- the assembler will automatically do the following:

- clean any old memory image files

- read your .gurt file

- separate the .text and .data sections

- convert each gurt instruction into its 8-bit machine code

- convert each data value into a memory word

- write two new files:

  - instruction_mem.hex: contains your compiled machine instructions

  - data_mem.hex: contains the initial memory contents from the .data section

- right-click the instruction memory component -> “load image” -> select instruction_mem.hex

- right-click the data memory component -> “load image” -> select data_mem.hex

- reset the cpu and begin execution

=====================================================================

architecture

the yo cpu is an 8-bit single-cycle processor

it contains:

- a program counter

- instruction memory

- an instruction splitter

- a control unit

- a register file with four general-purpose registers

- an alu that performs add and sub

- a data memory

all instructions are 8 bits wide

the cpu executes one instruction per clock cycle

the program counter increments by 1 after each instruction

the cpu supports:

add sub load store

=====================================================================

binary encoding

each yo instruction is 8 bits wide. the instruction fields are:

opcode: 2 bits

- allows up to 4 instructions

register fields (rd rs rt rb): 2 bits each

- because the cpu has 4 registers

immediate field (for load and store): 2 bits

- allows offsets from 0–3

add, sub

- format: add rd rs rt

- format: sub rd rs rt

encoding structure:

opcode (2 bits) + rd (2 bits) + rs (2 bits) + rt (2 bits)

example:

add r1 r2 r3

opcode = 00

rd = 01

rs = 10

rt = 11

machine code:

00 01 10 11

load

- format: load rd rb imm

encoding structure:

opcode (2 bits) + rd (2 bits) + rb (2 bits) + imm (2 bits)

example:

load r1 r0 2

opcode = 10

rd = 01

rb = 00

imm = 10

machine code:

10 01 00 10

store

- format: store rs rb imm

encoding structure:

opcode (2 bits) + rs (2 bits) + rb (2 bits) + imm (2 bits)

example:

store r3 r0 2

opcode = 11

rs = 11

rb = 00

imm = 10

machine code:

11 11 00 10

=====================================================================

jobs

brandon:
built the cpu and added the fun components, worked on the demo

zidan:
designed the gurt language, worked on the demo program, and wrote the user manual